Angus Combustion Analyser

angus.py  —  Application API Reference  —  v3

# Import

from angus import Angus, Sample

angus.py is the only file needed in notebooks. All register addresses, angfac conversions, and packet layout are handled internally. angus_regs.py is a low-level dependency -- do not import it directly.

# Quick Start

ol = Overlay('/home/xilinx/angus/angus_v3.bit')

a  = Angus(ol.<ip_block_name>)        # use ol.ip_dict.keys() to find name

a.startup_conf(crank_n_teeth=60, phase_ref_deg=180.0)

a.wait_for_sync()

a.runtime_conf(tdc_offset_deg=92.5, trig_decimation=5)

print(a.rpm, a.tdc_deg, a.is_synced)

samples = Angus.decode_buffer(dma_words)

tdc = [s.tdc_deg for s in samples]

Raw register access (escape hatch during development):

a.regs.read('ANGLE_ANGFAC')

a.regs.write('TDC_OFFSET', value)

# Constructor

| Name / Call | Description |
| --- | --- |
| Angus(ip_core) | ip_core: PYNQ IP object (e.g. overlay.angus_0). Use ol.ip_dict.keys() to find the correct block name. The underlying AngusRegs instance is accessible as a.regs for raw register access. |

# startup_conf()

Sets parameters that require a hardware reset. Resets the hardware and latches all startup registers immediately -- no separate apply() call needed. Only the arguments supplied are changed; all others retain their previous value (or the default on first call). runtime_conf() values are re-applied automatically after the reset.

| Argument | Default | Type | Description |
| --- | --- | --- | --- |
| crank_n_teeth | 60 | int | Total teeth including missing |
| crank_n_missing | 2 | int | Number of missing teeth |
| cam_n_teeth | 1 | int | Cam edges per 2 crank revolutions |
| phase_ref_deg | 180.0 | float | Expected cam edge in engine degrees (0–719.9). 0–359.9 = phase 0 (compression stroke). 360–719.9 = phase 1 (exhaust stroke). Phase bit is derived automatically. |
| phase_tol_deg | 30.0 | float | Detection window half-width in crank degrees |
| dma_buffer_size | 2 | int | Engine cycles per DMA buffer |
| src | 'crank' | str | Angle source: 'crank' or 'encoder' |
| ref | 'cam' | str | Phase reference: 'cam' or 'peak' |
| ang_sel | 'tooth' | str | Angle stream: 'tooth' or 'pll' |
| angle_interp | False | bool | Bresenham interpolation between teeth |
| crank_edge | 'rising' | str | 'rising' or 'falling' |
| cam_edge | 'rising' | str | 'rising' or 'falling' |
| enc_n_ppr | 96 | int | Encoder pulses per revolution |
| enc_ab_edge_sel | 0 | int | Encoder AB edge: 0=rising 1=falling 2=both |
| enc_z_edge_sel | 0 | int | Encoder Z edge: 0=rising 1=falling |

# runtime_conf()

Sets parameters that can be changed at any time during normal operation without resetting the hardware. Only the arguments supplied are updated.

| Argument | Default | Type | Description |
| --- | --- | --- | --- |
| tdc_offset_deg | 0.0 | float | Crank degrees from Z edge to engine TDC |
| trig_decimation | 1 | int | Sample rate divisor. 1 = every 0.1 deg equiv |
| max_rpm | 6000 | int | Speed fault threshold (RPM) |
| peak_hyst | 128 | int | Peak detector hysteresis counts |
| pll_kp | 0 | int | PLL proportional gain |
| pll_ki | 0 | int | PLL integral gain |
| pll_corr_max | 0xFFFF | int | PLL max correction NCO LSB |
| pll_corr_dir | 'add' | str | PLL correction direction: 'add' or 'subtract' |
| crank_dbc | 5 | int | Crank debounce cycles @ 100 MHz |
| cam_dbc | 5 | int | Cam debounce cycles |
| enc_a_dbc | 5 | int | Encoder A debounce cycles |
| enc_b_dbc | 5 | int | Encoder B debounce cycles |
| enc_z_dbc | 5 | int | Encoder Z debounce cycles |

# Control

| Name / Call | Description |
| --- | --- |
| a.clear_faults() | Clear all fault counters. |
| a.wait_for_sync(level, timeout) | Block until sync reaches level: 'moving', 'crank', or 'full' (default). timeout in seconds (default 10.0). Returns True if reached, False on timeout. |

# Status Properties

All properties read directly from hardware on each access.

| Field / Property | Type | Description |
| --- | --- | --- |
| a.rpm | int | Engine speed RPM (z-period based, low noise) |
| a.rpm_fast | int | Engine speed RPM (ab-period based, fast update) |
| a.tdc_deg | float | TDC-referenced engine angle, 0.0–719.9° |
| a.crank_deg | float | Raw crank angle, 0.0–360.0° (Z-referenced) |
| a.sync_state | str | 'STOPPED' / 'MOVING' / 'CRANK_SYNC' / 'FULL_SYNC' |
| a.is_synced | bool | True when FULL_SYNC |
| a.phase_found | bool | True after first successful cam detection |
| a.phase_ok | bool | True when cam detection is healthy |
| a.engine_phase | int | Current engine phase: 0 (compression) or 1 (exhaust) |
| a.faults | dict | Active fault flags and per-type counts (see below) |
| a.packet_count | int | DMA packets transmitted since last reset |
| a.overflow_count | int | DMA samples dropped (buffer not ready in time) |

## faults dict structure

{ 'cam': bool, 'crank': bool, 'ab': bool, 'speed': bool, 'pll': bool,

'counts': { 'cam': int, 'crank': int, 'pll': int,

'ab': int, 'speed': int, 'encoder': int } }

# Packet Decoding

| Name / Call | Description |
| --- | --- |
| Angus.decode(words) | Decode one 6-word DMA sample. words: sequence of 6 unsigned 32-bit integers. Returns a Sample namedtuple. Static method -- call on the class. |
| Angus.decode_buffer(buf, n_samples) | Decode an entire DMA buffer. buf: flat array of 32-bit words (6 per sample). n_samples: optional limit. Returns list of Sample namedtuples. |

## Sample namedtuple

| Field / Property | Type | Description |
| --- | --- | --- |
| s.tdc_deg | float | TDC-referenced engine angle, 0.0–719.9° |
| s.rpm_slow | int | z-period RPM |
| s.rpm_fast | int | ab-period RPM |
| s.di | int | Digital inputs byte (0–255) |
| s.adc | tuple[6] | ADC channels 0–5, 12-bit each. Access as s.adc[0]–s.adc[5] |

## Typical decode loop

words   = np.frombuffer(dma_buf, dtype=np.uint32)

samples = Angus.decode_buffer(words)

tdc  = np.array([s.tdc_deg for s in samples])

adc1 = np.array([s.adc[0]  for s in samples])

# Diagnostics

| Name / Call | Description |
| --- | --- |
| a.config_status() | Full readback of current configuration. Shows all startup and runtime parameter values from the internal shadow dicts in two sections. |
| a.run_status() | Runtime signal values read live from hardware, grouped by block: Sync, Speed, Angle, Phase, PLL, Fault, Pack. |

# Raw Register Access

The underlying AngusRegs instance is exposed as a.regs for direct register access during development, before all functionality has been wrapped in higher-level methods.

| Name / Call | Description |
| --- | --- |
| a.regs.read(name) | Read a named register. Returns unsigned field value. |
| a.regs.write(name, value) | Write a named register (field-masked, lsb-aligned). |
| a.regs.read_signed(name) | Read a named register as a signed 32-bit integer. |
| a.regs.read_raw(addr) | Read a 32-bit register by byte address. |
| a.regs.write_raw(addr, v) | Write a 32-bit register by byte address. |

Register names are defined in angus_regs.py. Examples:

a.regs.read('ANGLE_ANGFAC')

a.regs.read('SYNC_STATE')

a.regs.read_signed('PLL_ERR_ANGFAC')

# Overlay Load Errors

Exceptions raised by pynq.Overlay() before Angus is constructed:

| Name / Call | Description |
| --- | --- |
| OSError | Bitstream (.bit) or hardware handoff (.hwh) file not found. Both must exist with matching stem names in the same directory. |
| RuntimeError | Overlay metadata could not be parsed (mismatched .hwh), or an IP attribute is accessed before the overlay is loaded. |
| TimeoutError | PL programming interface timed out. Usually a kernel or driver issue. |

Find the correct IP block name:

print(ol.ip_dict.keys())

for name, info in ol.ip_dict.items(): print(f'{name:40s} {info["type"]}')