filter (was signal_conditioner):

- 2FF glitch filter to prevent metastability

- debouncer

axi config: debounce_count value for each of the 5 instances

inputs: raw

outputs: clean

cam (was cam_input):

- edge detection (rising or falling edge as per cam_edge_sel)

- counting of cam edges between z_edge pulses

axi config: cam_edge_sel (0 = falling edges, 1 = rising edges)

inputs: cam_clean
                 z_edge

outputs: cam_edge (positive pulse)
                    cam_tooth_count (per 2 crank revs)

crank (was crank_input):

- gap detection

  - timeout based on crank_gap_threshold

  - 1st edge following clears tooth count and starts z pulse

- tooth counting (real and real+interpolated)

- missing tooth interpolation

  - linear interpolation of missing teeth based on last tooth period

- tooth period calculation (per input tooth edge)

- real tooth counting (per rev)

axi config: crank_edge_sel (0 = falling edges, 1 = rising edges)
                        crank_gap_threshold (timeout as tooth period factor value)
                        crank_n_teeth (number of teeth total (inc missing))
                        crank_n_missing (number of missing teeth

inputs: crank_clean

outputs: crank_ab_edge (strobe from all ab edges)
                    crank_z_edge (strobe from z rising edges)
                    crank_ppr_conf (pass-through of crank_n_teeth to src_sel mux)
                    crank_tooth_period
                    crank_tooth_count (real tooth count per rev)
                    crank_ab_count (counting of all ab edges, including interpolated, per rev)
                    crank_gap_det
                    crank_ab (toggles on each tooth edge, including interpolated teeth)
                    crank_z (positive pulse starting on 1st edge after gap and finishing on next ab edge)

enc (was enc_input):

- tooth counting (3 counters: a, b and ab signals)

- correct or setting enc_signal_ok when correct

- a/b channel merging to create enc_ab output

- period calculation between ab edges

- edge detector for ab edges

axi config: enc_ab_edge_sel (rising, falling or both edge detection for a and b channels to create enc_ab)
                        enc_z_edge_sel (rising or falling z edge detection as 0deg)
                        enc_n_ppr (pulse-per-rev of encoder)

inputs: a_clean
                 b_clean
                 z_clean

outputs: enc_ab_edge (strobe from all ab edges)
                    enc_z_edge (strobe from all z edges)
                    enc_ppr_conf (pass-through of enc_n_ppr to src_sel mux)
                    enc_ab_period
                    enc_ab_count
                    enc_a_count
                    enc_b_count
                    enc_ab
                    enc_z

src_sel:

- when src_sel = 0:

  - ab_edge = crank_ab_edge

  - z_edge = crank_z_edge

  - ppr_conf = crank_ppr_conf

  - ab_period = crank_tooth_period

  - ab_count = crank_ab_count

- when src_sel = 1:

  - ab_edge = enc_ab_edge

  - z_edge = enc_z_edge

  - ppr_conf = enc_ppr_conf

  - ab_period = enc_ab_period

  - ab_count = enc_ab_count

axi config: src_sel

ref_sel:

- when ref_sel = 0:

  - ref_edge = cam_edge

- when ref_sel = 1:

  - ref_edge = peak_edge

axi config: ref_sel

angle (was angle_calc):

- linearly interpolation of engine angle (Bresenham method) – ab_edge and ab_period passed in from src_sel)

- 1st z_edge received sets angle to 0 deg, then every other z_edge sets to 0deg.

axi config:

inputs: ab_edge
                 z_edge
                 ab_period

outputs: angle_deg (16bit, 0-7199, 0.1deg/LSB)

phase (was phase_detector):

- phase_raw calc (0 when 0 <= angle_deg <= 3599, 1 when 3600 <= angle_deg <= 7199)

- phase_inv, phase_ref_ok, phase_inv_latch and phase_ref_found all default to 0

- engine angle tracking:

  - phase_ang_eng held at 0 until phase_ref_found = 1

  - phase_ang_corr  = angle_deg + (phase_inv_latch * 3600) % 7200

  - phase_ang_eng = (phase_ang_corr + tdc_offset) % 7200

- window detection:

  - finds angle phase_ang_corr when ref_edge received – phase_ang_corr is shifted is phase_inv_latch = 1, moving it to window 1 if it occurs in window 2.

  - window 1 = phase_ref_ang +- phase_ref_ang_tol

  - window 2 = (window 1 + 360deg) % 720deg

  - if ref_edge occurs in window 1, phase_inv = 0 and phase_ref_det = 1

  - if ref_edge occurs in window 2, phase_inv = 1 and phase_ref_det = 1

  - the first time phase_ref_det = 1 phase_inv is latched into phase_inv_latch and phase_ref_found is latched to 1 – any further occurrence of ref_edge should be in window 1

  - phase_eng = phase_raw XOR phase_inv_latch

  - each occurrence of phase_ref_det increments phase_ref_det_count

  - if phase_ref_found = 1:

    - when phase_ref_det = 1 and phase_inv = phase_inv_latch, phase_ref_ok = 1 (registered)

    - when phase_ref_det = 1 and phase_inv != phase_inv_latch, phase_ref_ok = 0 (registered)

    - if 3x z_edge pulses are received without phase_ref_det_count incrementing, phase_ref_ok = 0 (registered)

axi config: phase_ref_ang (expected position of the reference pulse in angle_deg)
                        phase_ref_ang_tol (+- tolerance on the above)
                        tdc_offset (tdc offset from crank 0deg to engine TDC)

inputs: ref_edge (from ref_sel)
                 angle_deg (from angle)
                 z_edge

outputs: phase_raw (phase of angle_deg input – 0 or 1)
                    phase_ref_det (ref_edge has been detected in either window)
                    phase_ref_ok (ref_edge has been detected on correct phase)
                    phase_ref_found (latched to 1 first time phase_ref_ok = 1)
                    phase_inv (0 and phase_ref_ok means ref_edge is in expected window)
                    phase_inv_latch (phase inv latched at first ref_edge detection)
                    phase_ang_corr (angle applied by phase_inv_latched to move ref_edge to the correct phase) 
                    phase_eng (engine phase – 0 or 1)
                    phase_ang_eng (engine angle with phase correction and tdc offset applied)

sync (was sync_fsm):

- FSM that tracks sync state

  - (0) STOPPED: ab_count = 0 and no z_edge received

  - (1) MOVING: ab_count > 0, but 2 z_edges not yet received

  - (2) CRANK_SYNC: 2 z_edges received and ab_count = ppr_conf at 2nd z_edge

  - (3) FULL_SYNC: CRANK_SYNC + phase_ref_found

axi config:

inputs: phase_ref_found
                 z_edge (from src_sel)
                 ppr_conf (from src_sel)
                 ab_count (from src_sel)

outputs: sync_state
                    sync_full (=1 when sync_state = 3)
                    sync_fault_count

speed:

- Calculates 2 engine speed values:

  - Slow: from time between z_edges, updated every z_edge once 2 received

    - speed_rpm_slow = 60000 / z_period(ms)

  - Fast: from ab_period, updated every ab_edge

    - speed_rpm_fast = 60000 / (ab_period(ms) * ppr_conf)

axi config:

inputs: ab_period
                 ab_edge
                 z_edge
                 ppr_conf

outputs: speed_rpm_slow
                    speed_rpm_fast

pll (was angle_engine):

- NCO creates hi-res angle value that’s most accurate that angle_deg to enable sampling at small angle increments

- PI loop calculates at each ab_edge – i-term calculation ab_period at the integration dt to compensate for different frequency of updates due to varying engine speed.

- Calculates nco_inc and each ab_edge (divider successful operation tracked with div_valid)

- nco or PI loop aren’t allowed to start until sync_full = 1

- pll_ang_hires held at 0 until sync_full = 1

- pll_nco_ab_inc = (2^32) / prr_conf, calculated during config_apply

- pll_cycle_ab_count: increments on change in ab_count and resets every 2nd z_edge (counts to ppr_conf * 2 every 2 revolutions before reset to 0)

- pll_phase_err = pll_nco_accum – (ab_count * pll_nco_ab_inc)

axi config: pll_kp
                        pll_ki
                        pll_corr_dir
                        pll_corr_max

inputs: sync_full
                 phase_eng
                 ab_count (from src_sel)
                 ab_edge (from src_sel)                
                 ab_period (from src_sel)
                 z_edge (from src_sel)

outputs: pll_ang_hires (16bit, 0-7199, 0.1deg/LSB)
                    pll_div_valid
                    pll_nco_inc
                    pll_nco_accum
                    pll_phase_err
                    pll_p_term
                    pll_i_term
                    pll_pi_corr (signed value)
                    pll_cycle_ab_count

trig (was sample trigger):

- ang_deg != ang_deg_prev increments and internal compare counter

- after counter value matches trig_decimation, counter is reset and trig_pulse is sent

- Outputs a trig_pulse with trig_pulse_width (number of cycles)

- trig_pulse_count increments on each trig_pulse and resets on each z_edge

axi config: trig_decimation
                        trig_pulse_width

inputs: ang_deg (16bit, 0-7199, 0.1deg/LSB)
                 z_edge (from sec_sel)

outputs: trig_pulse
                    trig_pulse_count

ang_sel:

- Chooses between interpolated engine angle, phase_eng_ang and PLL engine angle, pll_ang_hires

axi config: ang_sel

inputs: phase_eng_ang
                 pll_ang_hires

outputs: ang_deg

pack (was sample_packer):

- tlast on last word every 2 cycles (1440 deg)

- double-buffer DMA (2 cycles per buffer)

- buffer size can be changed using axi reg DMA_BUFFER_SIZE and these steps:

  - PS stops processing

  - PS reallocates DMS buffers at new size

  - PS write new size to DMA_BUFFER_SIZE reg

  - PS sets config_apply (auto-clearing)

  - PS resubmits DMA descriptors with new buffer size

packet format:
Word 0: [31:16] ang_deg [15:0] speed_rpm_fast
Word 1: [31:16] DI[7:0] + pad [15:0] ADC_ch1 (12-bit, zero-padded)
Word 2: [31:16] ADC_ch2 [15:0] ADC_ch3
Word 3: [31:16] ADC_ch4 [15:0] ADC_ch5
Word 4: [31:16] ADC_ch6 [15:0] reserved

12bit xadc padding: "0000" & adc_data(11 downto 0)

axi config:

inputs: DI ch1-8
                 ADC ch1-6
                 ang_deg
                 speed_rpm_fast

outputs: AXI DMA stream

fault

- Performs centralised fault detection for the whole system

- fault detection ever 2 z_edges after the first 2 z_edges have been received

  - cam_tooth_count = cam_n_teeth

    - instantaneous fault: cam_signal_fault

    - fault count: cam_fault_count

- fault detection at each z_edge after 2 z_edges have been received

  - crank_tooth_count /= (crank_n_teeth – crank_n_missing)

    - instantaneous fault: crank_tooth_fault

    - fault count: crank_fault_count

  - crank_ab_count /= crank_n_teeth

    - instantaneous fault: crank_ab_fault

    - fault count: crank_fault_count (shared with crank_tooth_fault)

- on every falling edge of phase_ref_ok, increment phase_fault_count

- when sync_state = 2 or 3, check that ab_count = ppr_conf at each z_edge – ab_fault_count incremented if not.

- speed_rpm_slow range-checked – if < 0 or > 6000 at z_edge, increment speed_fault_count and set speed_fault

- check the PLL PI controller doesn’t get too far from the phase

  - abs(pll_phase_error) > fault_pll_phase_thresh

axi config: cam_n_teeth (number of cam teeth per 2 crank revs)	
                        crank_n_teeth
                        crank_n_missing

inputs: z_edge (from src_sel)
                cam_edge_count
                crank_tooth_count
                crank_ab_count
                phase_ref_ok
                sync_state
                ppr_conf (from src_sel)
                speed_rpm_slow
                pll_phaser_err

outputs: fault_cam_signal
                    fault_cam_count
                    fault_crank_tooth 
                    fault_crank_ab 
	    fault_crank_count
                    fault_phase_err
                    fault_phase_count
                    fault_ab_count
                    fault_speed_calc
                    fault_speed_count

axi_lite_regs:

- 9-bit addressing (512 bytes)

- Register addresses and contents defined in “AXI Register Map (target architecture)” section

Debugging Digital Ouputs (signals already outputted by blocks that just need wiring to output ports in top.vhd)

- W18 - crank_clean

- W19 - cam_clean

- Y18 - crank_ab_edge (widened to 1us pulse)

- Y19 - ref_edge (widened to 1us pulse)

- U8 – ab_edge (widened to 1us pulse)

- W6 – z_edge (widened to 1us pulse)

- B20 - crank_gap_det

- W8 – phase_ref_det

- W10 - trig_pulse

- B19 - pll_div_valid

- V8 - crank_signal_ok

- C20 - sync_full

- V6 - phase_inv_latch

- V10 – phase_ref_found

- F20 –

- F19 –

- W16 –

- Y16 –

- Y17 –

- V16 –

- W9 –

- A20 –

Notes

- all edge detect output signals are 1 clock period in duration

Signal Types and Conventions

All signals are synchronous to clk (100 MHz), except the AXI-Lite interface which uses s_axi_aclk.

The angle domain: 16-bit unsigned, 0-7199, 0.1 deg per LSB (7200 counts = 720 deg = one 4-stroke cycle).

Edge strobes (_edge, _pulse suffixes):  std_logic, exactly 1 clock wide

Angle values (angle_deg, pll_ang_hires, phase_ang_eng etc):  unsigned(15 downto 0), 0-7199, 0.1 deg/LSB

ab_period, tooth_period, gap_period:  unsigned(31 downto 0), clock cycles @ 100 MHz

ppr_conf, ab_count, tooth counts:  unsigned(7 downto 0)

Fault counters, speed (RPM):  unsigned(15 downto 0)

sync_state:  unsigned(1 downto 0)  0=STOPPED  1=MOVING  2=CRANK_SYNC  3=FULL_SYNC

PLL phase error, p_term, i_term, pi_corr:  signed(31 downto 0), NCO accumulator units

Single-bit flags (phase_raw, phase_eng, faults etc):  std_logic

Reset Behaviour

rst = (not rst_n) OR (not config_valid). PL held in reset until PS sets config_apply = 1(self-resetting), then a delay of RST_CYCLES (defaults to 256). config_apply latches startup config registers.

Setting config_apply during operation updates config latch registers from axi write registers, then applies the same RST_CYCLES delay before releasing reset.

AXI registers accessible during reset. All blocks reset to safe defaults on rst.

Startup config (latched on config_apply):

crank_edge_sel
crank_n_teeth
crank_n_missing
crank_gap_threshold
cam_edge_sel
cam_n_teeth
enc_ab_edge_sel
enc_z_edge_sel
enc_n_ppr
src_sel
ref_sel
ang_sel

Startup Calcs (immediately after startup config is latched):

- plll_nco_ab_inc

  - if src_sel = 0:
    pll_nco_ab_inc = 2^32 / crank_n_teeth

  - if src_sel = 1:
    pll_nco_ab_inc = 2^32 / enc_n_ppr

Runtime config (direct from register, update any time):

all debounce counts
pll_kp
pll_ki
pll_corr_dir
pll_corr_max
pll_phase_err_thresh
phase_ref_ang
phase_ref_ang_tol
tdc_offset
trig_decimation
trig_pulse_width
fault_clear

AXI Register Map (target architecture)

Write Registers (PS to PL)

0x00  CONTROL  [31:0] [all startup]
 [0]  crank_edge_sel         0=falling, 1=rising (crank) 
 [1]  src_sel                0=crank, 1=encoder
 [2]  ref_sel                0=cam, 1=peak_detector
 [3]  config_apply           self-clearing, latches startup config, releases rst
 [4]  cam_edge_sel           0=falling, 1=rising (cam)
 [5]  ang_sel                0=angle_deg (tooth-based), 1=pll_ang_hires

0x04 CONTROL_RT [31:0]
[0] fault_clear – self-clearing pulse
[1] pll_corr_dir – 0=subtract, 1=add
[2] phase_fault_drop – 0=count only , 1=drop to CRANK_SYNC on fault

0x08 RST_CYCLES   [8:0]  clock cycles after reset asserted before self-release (default 256)

filter config
0x0C  CAM_DBC              [15:0]     cam filter debounce cycles  default 5 (50ns)  [runtime]
0x10  CRANK_DBC            [15:0]     crank filter debounce cycles  default 5 (50ns)  [runtime]
0x14  ENC_A_DBC            [15:0]     encoder A filter debounce cycles  default 5 (50ns)  [runtime]
0x18  ENC_B_DBC            [15:0]     encoder B filter debounce cycles  default 5 (50ns)  [runtime]
0x1C  ENC_Z_DBC            [15:0]     encoder Z filter debounce cycles  default 5 (50ns)  [runtime]

crank config
0x20  CRANK_GAP_THRESH     [7:0]      gap threshold, 1.7 fixed point (0xC0 = 1.5x tooth period)  default 0xC0  [startup]
0x24  CRANK_N_TEETH        [7:0]      total crank teeth including missing  default 60  [startup]
0x28  CRANK_N_MISSING      [7:0]      number of missing teeth  default 2  [startup]

cam config
0x2C  CAM_N_TEETH          [7:0]      cam teeth per 2 crank revolutions  default 1  [startup]

enc config
0x30  ENC_N_PPR            [15:0]     encoder pulses per revolution  [startup]
0x34  ENC_AB_EDGE_SEL      [1:0]      0=rising, 1=falling, 2=both edges for a and b  default 0  [startup]
0x38  ENC_Z_EDGE_SEL       [0]        0=rising, 1=falling z edge  default 0  [startup]

phase config
0x3C  PHASE_REF_ANG        [15:0]     expected ref pulse position, 0-7199, 0.1 deg  [runtime]
0x40  PHASE_REF_TOL        [15:0]     window tolerance +/-, 0-7199, 0.1 deg  [runtime]
0x44  TDC_OFFSET           [15:0]     TDC offset from crank 0deg to engine TDC, 0-7199, 0.1 deg  [runtime]
0x48 PLL_PHASE_ERR_THRESH [31:0] max abs PLL phase error before fault, NCO units [runtime]

pll config
0x4C  PLL_KP               [15:0]     PLL proportional gain  default 0  [runtime]
0x50  PLL_KI               [15:0]     PLL integral gain  default 0  [runtime]
0x54  PLL_CORR_MAX         [15:0]     PLL max correction, NCO LSB units  [runtime]

trig config
0x58  TRIG_DECIMATION      [15:0]     decimation count (1=every 0.1deg, 10=every 1deg)  default 1  [runtime]
0x5C  TRIG_PULSE_WIDTH     [15:0]     trigger pulse width, clock cycles (1000=10us @ 100MHz)  default 1000  [runtime]

pack config
0x60 DMA_BUFFER_SIZE   [3:0] (default 2)  number of engine cycles (720deg) to transfer each DMA request [startup]

Read Registers (PL to PS)

sync
0x70  SYNC_STATE           [1:0]      0=STOPPED 1=MOVING 2=CRANK_SYNC 3=FULL_SYNC  [read]
0x74  SYNC_FAULT_COUNT     [15:0]     sync fault event count  [read]

speed
0x78  SPEED_RPM_SLOW       [15:0]     RPM from z_period (updated every z_edge)  [read]
0x7C  SPEED_RPM_FAST       [15:0]     RPM from ab_period (updated every ab_edge)  [read]

angle
0x80  ANGLE_DEG            [15:0]     tooth-based interpolated angle, 0-7199, 0.1 deg  [read]

phase
0x84  PHASE_RAW            [0]        phase of angle_deg: 0=first rev (0-3599), 1=second rev (3600-7199)  [read]
0x88  PHASE_REF_DET        [0]        ref_egde detected in either window (strobe)  [read]
0x8C  PHASE_REF_OK         [0]        ref_edge detected in correct window (registered)  [read]
0x90  PHASE_REF_FOUND      [0]        latched: 1 after first valid ref detection  [read]
0x94  PHASE_INV            [0]        ref_edge landed in window 2 (wrong phase)  [read]
0x98  PHASE_INV_LATCH      [0]        phase_inv latched at first detection  [read]
0x9C  PHASE_ANG_CORR       [15:0]     phase-corrected angle, 0-7199, 0.1 deg  [read]
0xA0  PHASE_ENG            [0]        engine phase = phase_raw XOR phase_inv_latch  [read]
0xA4  PHASE_ANG_ENG        [15:0]     engine angle with phase+TDC correction, 0-7199, 0.1 deg  [read]
0xA8  PHASE_REF_DET_CNT    [15:0]     count of ref pulse detections  [read]

pll
0xAC  PLL_ANG_HIRES        [15:0]     PLL NCO angle, 0-7199, 0.1 deg  [read]
0xB0  PLL_DIV_VALID        [0]        divider operation valid flag  [read]
0xB4  PLL_NCO_INC          [31:0]     NCO frequency word  [read]
0xB8  PLL_NCO_ACCUM        [31:0]     NCO accumulator (raw 32-bit value)  [read]
0xBC  PLL_PHASE_ERR        [31:0]     signed phase error, NCO units  [read]
0xC0  PLL_P_TERM           [31:0]     signed proportional term  [read]
0xC4  PLL_I_TERM           [31:0]     signed integral term  [read]
0xC8  PLL_PI_CORR          [31:0]     signed PI correction applied to NCO  [read]
0xCC PLL_NCO_AB_INC  [31:0]   nco count increment per ab edge to create expect angle in pll
0xD0 PLL_CYCLE_AB_COUNT  [31:0]  ab_edge count every cycle (2 revs)

trig
0xD4  TRIG_PULSE_COUNT     [31:0]     total trigger pulses generated  [read]

crank readback
0xD8  CRANK_TOOTH_PERIOD   [31:0]     last tooth period, clock cycles @ 100 MHz  [read]
0xDC  CRANK_GAP_PERIOD         [31:0]     last gap period at detection, clock cycles @ 100 MHz  [read]
0xE0  CRANK_TOOTH_COUNT    [7:0]      real tooth count per revolution  [read]
0xE4  CRANK_AB_COUNT              [7:0]      all ab edges per revolution including interpolated  [read]
0xE8  CRANK_GAP_DET                  [0]        gap currently detected  [read]

cam readback
0xEC  CAM_TOOTH_COUNT      [7:0]      cam edge count per 2 crank revolutions  [read]
0xF0  REF_ANGLE                         [15:0]     angle at last ref pulse detection, 0-7199, 0.1 deg  [read]

enc readback
0xF4  ENC_AB_COUNT          [7:0]      encoder ab edge count  [read]
0xF8  ENC_A_COUNT             [7:0]      encoder a channel edge count  [read]
0xFC  ENC_B_COUNT             [7:0]      encoder b channel edge count  [read]
0x100  ENC_AB_PERIOD         [31:0]     encoder ab period, clock cycles  [read]

fault readback
0x104 FAULT_FLAGS	                   [31:0] instantaneous fault flags [read] 
    [0] cam_signal_fault 
    [1] crank_tooth_fault
    [2] crank_ab_fault
    [3] speed_fault
    [4] pll_phase_err_fault
    [5:31] reserved
0x108  CAM_FAULT_COUNT 	    [15:0] cam tooth count fault events [read]
0x10C CRANK_FAULT_COUNT 	    [15:0] crank fault count (tooth+ab shared) [read]
0x110  PHASE_FAULT_COUNT 	    [15:0] phase_ref_ok falling edge count [read]
0x114 AB_FAULT_COUNT 	    [15:0] ab count mismatch at z_edge [read]
0x118  SPEED_FAULT_COUNT 	    [15:0] speed fault event count [read]
0x11C  PLL_PHASE_ERR_COUNT  [15:0] PLL phase error exceeded thresh count [read]

pack readback
0x120  PKT_COUNT            [31:0]     DMA packet count  [read]
0x124  OVF_COUNT           [15:0]     DMA overflow count  [read]